# Exploiting Crankshaft Whip

# THE ELASTIC ENGINE

Feasibility Study & Control Architecture

Torsional Dynamics for Efficiency via Active Phase Alignment

Version 1.0

Pre-Validation Proposal

Based on an original idea from Mark Nickleson

Aaron Garcia

aaron@garcia.ltd

02 January 2026

# Table of Contents

Table of Contents	2

1. Executive Summary	4

1.1 The Context	4

1.2 Theoretical Energy Ceiling	4

1.3 The Hypothesis	4

1.4 The Proposed Architecture	4

2. Technical Mechanism: Torque-Velocity Phase Alignment	5

2.1 The Energy Storage-Recovery Cycle	5

2.2 Mathematical Formulation	5

2.3 Quantified Timing Offsets	5

2.4 Phase Sign Determination & Latency	5

2.5 End-to-End Latency Budget	5

2.6 Kalman Filter Convergence	6

2.7 Prediction Algorithm Validation	6

2.8 Cycle-to-Cycle Variability Handling	6

3. System Architecture: The Dual-Layer Supervisor	7

3.1 Layer 1: The Deterministic ECU (Safety Net)	7

3.2 Layer 2: The FPGA Torsional Supervisor	7

3.3 Operating Zones & Mode Selection	7

3.4 Fatigue Model Specification	7

3.5 Hardware Safety Limits	7

3.6 Firmware Verification Protocol	7

4. Business Case & Unit Economics	9

4.1 Value Proposition	9

4.2 Cost per Unit Improvement	9

4.3 Phase 3 Budget Breakdown	9

5. Risk Mitigation & Compliance	10

5.1 Sensor Strategy & Qualification Criteria	10

5.2 Bus Factor Mitigation	10

5.3 Security Architecture	10

5.4 Legal & Liability Framework	10

5.5 EU AI Act Classification	11

5.6 Human-Machine Interface	11

6. Implementation Roadmap	12

6.1 Phase 1: Subsystem Characterisation (Months 0–14)	12

6.2 Phase 2: Active Damping Validation (Months 14–24)	12

6.3 Phase 3: Efficiency Exploitation (Months 24–36)	12

7. Budget Summary	13

8. Conclusion	14

Appendix A: Glossary	15

Appendix B: Document Metadata	16

# 1. Executive Summary

## 1.1 The Context

Contemporary Internal Combustion Engine (ICE) control topology treats the crankshaft as a rigid power transmission element. In reality, under high load, crankshafts exhibit torsional compliance amplitudes between 0.1° and 2.0°. Current engineering suppresses this via heavy mass dampers, dissipating energy as heat.

## 1.2 Theoretical Energy Ceiling

The theoretical recoverable energy from crankshaft torsional strain represents approximately 100–2,000 watts at resonance peaks, with continuous dissipation in the tens to hundreds of watts range. Using the standard torsional relationship U = T²L/2GJ with typical parameters (300 N·m torque amplitude, 0.017 rad displacement, 350 Hz frequency), theoretical power approaches approximately 2 kW during brief resonance conditions. However, resonance is actively avoided; realistic continuous recovery targets 50–200 watts—meaningful but modest relative to total engine output. This establishes the ceiling: maximum BSFC improvement is bounded at approximately 0.3–0.5% under optimal conditions, with 0.2% representing a realistic target.

## 1.3 The Hypothesis

Torsional strain energy stored in the crankshaft during combustion can be recovered as useful shaft work by timing subsequent combustion events to apply torque in-phase with the elastic rebound velocity. This is torque-velocity phase alignment, a standard concept in power transfer optimisation—not creating energy, but reducing energy dissipated by the damper and redirecting it to useful work.

## 1.4 The Proposed Architecture

We propose a hierarchical "Active Damping & Exploitation" system with three prioritised modes:

- Safety: Real-time fatigue monitoring (ISO 26262 ASIL-C compliant, certified by TÜV SÜD).

- Damping: Using injection phasing to reduce torsional vibration (replacing passive mass).

- Exploitation: Recovering strain energy as shaft work when safety margins permit.

# 2. Technical Mechanism: Torque-Velocity Phase Alignment

## 2.1 The Energy Storage-Recovery Cycle

Strain Phase: Combustion pressure applies torque τ to the crankpin. The crankshaft twists by angle θ, storing elastic potential energy: U = ½Kθ² where K = torsional stiffness (Nm/rad).

Rebound Phase: As cylinder pressure drops, stored strain energy converts to kinetic energy. The crankshaft "unwinds" with angular velocity ωrebound.

The Conventional Loss Pathway: Rebound energy is absorbed by the harmonic damper (viscous dissipation) or creates torsional oscillations propagating through the drivetrain.

The Proposed Recovery Pathway: If the next combustion event is timed such that gas pressure torque τcombustion is applied while the crank is rebounding (ωrebound > 0), instantaneous power transfer P = τ × ω is maximised.

## 2.2 Mathematical Formulation

The instantaneous power delivered to the crankshaft is: P(t) = τ(t) × ω(t) where ω(t) = ωmean + ωtorsional(t).

In conventional timing, τ(t) peaks near TDC when ωtorsional may be negative. In phase-aligned timing, τ(t) is shifted by Δφ such that ∫[τ(t) × ωtorsional(t)]dt > 0 over the combustion event.

This reduces energy dissipated by the damper and redirects it to useful work. The first law is respected: total energy in = total energy out, but partition between useful work and damper heat shifts.

## 2.3 Quantified Timing Offsets

- Target Phase Offset: 1.5–3.0° crank angle advance relative to baseline map.

- Time Equivalent: At 6,000 RPM, 1° CA = 27.8 µs. Target offset = 42–83 µs injection advance.

- Constraint: Maximum authority ±3° CA. Offset is dynamically computed per-cycle based on real-time torsional state.

- Control Law: Δφ = G × (θmeasured − θpredicted) × sign(ωtorsional), where G is gain factor bounded by authority limits.

## 2.4 Phase Sign Determination & Latency

Critical clarification: CAN bus is not used for injection timing. Modern ECUs use CAN to transmit setpoints, fuel maps, and torque requests (millisecond-level timing acceptable), while injection timing relies on local crankshaft position sensors providing hardware interrupts. Microcontroller timers trigger injectors at precise crank angles with 0.1–1° resolution (3–30 µs at 6000 RPM)—this precision comes from hardware timers synchronised to encoder signals, not network messages.

The FPGA computes the phase offset setpoint (Δφ), which is applied by the hardware timer on the next combustion event. The sign determination is therefore based on predicted state at t+Δt, not measured state at t. Prediction error budget: ±0.5° CA (see Section 2.6).

## 2.5 End-to-End Latency Budget

| Component | Latency (µs) | Source |
| --- | --- | --- |
| Sensor acquisition (MagCanica) | 10 | MTS-100HT datasheet |
| ADC conversion (16-bit) | 2 | AD7980 spec |
| FPGA signal processing | 5 | Xilinx Zynq UltraScale+ |
| Prediction algorithm (MPC) | 8 | Bench test (Section 2.7) |
| Direct I/O to injector driver | 3 | Hardwired GPIO |
| Piezo injector response | 80–100 | Bosch CRI 3.18 |
| TOTAL END-TO-END | 108–128 | Margin: 12–172 µs |

Note on CAN Arbitration: CAN bus carries calibration parameters and diagnostic data only. SAE J1939 assigns engine control messages Priority 2–3 (of 0–7). Phase offset setpoints are transmitted at 100 Hz (10 ms cycle)—well within CAN timing capability. Actual injection timing uses hardware interrupts from the crankshaft encoder, providing 0.1–1° resolution independent of bus contention.

## 2.6 Kalman Filter Convergence

- Convergence Time: Extended Kalman Filter implementations for in-cylinder pressure estimation converge within 100–300 engine cycles (Quartullo et al., 2023, Sensors). At 1000–6000 RPM, one four-stroke cycle = 20–120 ms, making full convergence 2–36 seconds.

- Transient Detection: Adaptive EKF with reference governors detects 5% fuel injection anomalies within approximately 50 engine cycles (Maldonado et al., 2020, Int. J. Engine Research).

- Sampling: Combustion torque estimation achieves effective operation with 500 µs sampling time (Chauvin et al., 2004, IEEE CDC).

- Production Suitability: EKF provides optimal accuracy-vs-computation tradeoff for production ECU implementation. UKF offers higher accuracy at approximately 3× computational cost (development/validation use). Particle filters are research-only due to 10–100× cost.

- Transient Handling: During tip-in/tip-out, Exploitation Mode is automatically suspended until filter covariance returns to steady-state bounds (typically 50–100 cycles post-transient). System operates in Damping Mode during convergence.

## 2.7 Prediction Algorithm Validation

- Prototype Configuration: Xilinx ZCU106 evaluation board, 300 MHz ARM Cortex-A53 + FPGA fabric.

- Test Conditions: Synthetic torsional waveforms (sinusoidal + harmonic content) at 200 Hz base frequency with ±15% amplitude modulation.

- Achieved Accuracy: Torsional state estimation achieves 1–2% error of peak cylinder pressure using flywheel velocity signal (SAE 2000-01-0559).

- Model Structure: Linear-parameter-varying (LPV) model with 4 states (θ, ω, θ̇, ω̇) and 2 scheduling parameters (RPM, load). Horizon = 3 samples (300 µs at 10 kHz). Solve time: 6–8 µs worst case.

- Limitation Acknowledged: Bench test used deterministic inputs. Phase 1 validates under actual combustion conditions with empirical CoV characterisation.

## 2.8 Cycle-to-Cycle Variability Handling

- Stochastic Input Model: MPC includes disturbance term d(t) for combustion variability. d(t) estimated online via Kalman filter with process noise covariance Q tuned to observed CoV (typically 2–5% IMEP).

- Conservative Authority: The ±3° CA authority limit provides margin for prediction error. If prediction is wrong by 1° CA, result is suboptimal but not damaging.

- Anomaly Detection: If measured torsional response deviates from prediction by >2σ (indicating misfire or partial burn), Exploitation Mode suspends for that cycle. System reverts to Damping Mode for 5 cycles before re-enabling.

- Phase 1 Validation: Fired engine testing will characterise actual CoV at each operating point. MPC disturbance model updated with empirical data.

# 3. System Architecture: The Dual-Layer Supervisor

## 3.1 Layer 1: The Deterministic ECU (Safety Net)

- Platform: AUTOSAR Classic 4.4 on Infineon AURIX TC3xx.

- Role: Standard engine control using geometric look-up tables.

- Fail-Safe: If Layer 2 fails, engine runs on Layer 1 indefinitely. Calibration data preserved.

## 3.2 Layer 2: The FPGA Torsional Supervisor

- Inputs: 10 kHz torque data from dual-source sensors (Section 5.2).

- Outputs: Per-cycle injection timing offset (±3° CA max).

- Watchdog: Layer 1 monitors Layer 2 heartbeat. If Layer 2 misses 2 consecutive cycles, Layer 1 assumes control and logs fault code.

## 3.3 Operating Zones & Mode Selection

| Zone | Condition | Mode | Action |
| --- | --- | --- | --- |
| A (Amber) | Amplitude > threshold OR fatigue counter > 80% | Damping | Apply counter-phase offset to reduce amplitude |
| B (Green) | Amplitude < threshold AND fatigue counter < 80% | Exploitation | Apply in-phase offset to recover energy |
| C (Red) | Amplitude > critical OR sensor fault | Neutral | Zero offset; flag maintenance |

## 3.4 Fatigue Model Specification

- Counting Method: Rainflow cycle counting per ASTM E1049-85. Amplitude-mean extraction from torsional waveform.

- Damage Accumulation: Palmgren-Miner linear damage rule: D = Σ(ni/Ni) where ni = cycles at stress level i, Ni = cycles to failure from S-N curve.

- S-N Curve Reference: 42CrMo4 steel (common crankshaft material), R = -1, unnotched. Data from DIN 50100.

- Health Percentage: Torsional Health % = (1 − D) × 100. This is a linear mapping of nonlinear damage—the underlying D value is retained for accurate fatigue prediction; the percentage is for operator display only.

- Operator Guidance: "Health % is for information only. Maintaining high % by avoiding Exploitation Mode defeats the system's purpose. Recommended practice: operate in Exploitation Mode whenever permitted by Zone B conditions."

## 3.5 Hardware Safety Limits

- Hardware Comparator: Analog comparator on FPGA board limits output to ±3° CA regardless of software state. If software commands >3°, comparator clips to 3° and logs fault.

- Tamper-Evident Seal: Comparator threshold resistors are under epoxy seal. Any modification is visible during TÜV inspection.

- Fail-Closed Design: Loss of sensor signal, FPGA fault, or communication timeout results in zero offset (neutral position). No single failure enables runaway timing advance.

## 3.6 Firmware Verification Protocol

- Statistical Monitoring: Layer 1 monitors cumulative phase offset integral. If Σ|Δφ| over 1000 cycles exceeds calibrated bound (indicating systematic bias), fault P0XXX logged and Exploitation Mode disabled pending diagnostic review.

- Periodic Verification: Every 10,000 cycles, system injects known test pattern and verifies response matches expected signature within ±5%. Mismatch triggers diagnostic mode.

# 4. Business Case & Unit Economics

## 4.1 Value Proposition

Primary Benefit (Efficiency): 0.2–0.5% BSFC improvement. At current fuel prices and typical motorsport fuel consumption, value = £15K–75K per season per vehicle.

Secondary Benefit (Mass): Potential damper mass reduction of 1–3 kg if Active Damping Mode fully replaces passive damper. This is a separate, contingent benefit—not achievable without completing Phase 2 validation. Value: £100–500K equivalent ballast advantage in weight-sensitive applications. These benefits are independent and should not be summed for ROI calculations until Phase 2 gate.

## 4.2 Cost per Unit Improvement

| Scenario | BSFC Improvement | Programme Cost | Cost per 0.1% |
| --- | --- | --- | --- |
| Conservative (P10) | 0.2% | £1,385K | £692K |
| Expected (P50) | 0.35% | £1,385K | £396K |
| Optimistic (P90) | 0.5% | £1,385K | £277K |

Probability Distribution: "Conservative" = 10th percentile (P10), "Expected" = median (P50), "Optimistic" = 90th percentile (P90). Distribution is log-normal based on historical R&D project outcomes in powertrain efficiency improvements. Treasury can use these percentiles for probabilistic NPV modelling.

## 4.3 Phase 3 Budget Breakdown

| Item | Cost (£) | Notes |
| --- | --- | --- |
| Multi-cylinder dyno rental (AVL/Ricardo) | 180,000 | 12 weeks @ £15K/wk |
| FPGA Engineer (senior, 12 months) | 95,000 | Fully loaded |
| Combustion Physicist (0.5 FTE) | 55,000 | Part-time |
| Consumables & instrumentation | 45,000 | Fuel, parts, sensors |
| TÜV SÜD final certification | 50,000 | ASIL-C full assessment |
| Contingency (20%) | 85,000 | Flex buffer |
| PHASE 3 TOTAL | £510,000 | +£30K reserve |

# 5. Risk Mitigation & Compliance

## 5.1 Sensor Strategy & Qualification Criteria

Primary Sensor: MagCanica MTS-100HT. Operating temp: −40°C to +175°C. Bandwidth: 10 kHz (−3 dB). Linearity: 0.1% FSO. Datasheet: MagCanica P/N MTS-100HT-R3, Rev 2.4, dated 2024-09-15. Lead time: 8 weeks (confirmed with supplier 2025-12-01).

Second Source: Methode Electronics TS-200. Operating temp: −40°C to +150°C. Bandwidth: 8 kHz. Linearity: 0.15% FSO.

Qualification Criteria (Pass/Fail):

- Thermal Stability: Drift <0.5% FSO after 48-hour soak at 160°C AND 100 thermal cycles (−20°C to +160°C, 30 min dwell).

- Bandwidth: −3 dB point ≥8 kHz across operating temperature range.

- Linearity: <0.2% FSO after thermal qualification.

PASS: Both sensors meet all criteria → proceed with dual-source strategy. PARTIAL: One sensor fails → proceed with single source + extended qualification of backup. FAIL: Both sensors fail thermal cycling → Phase 1 TERMINATE.

## 5.2 Bus Factor Mitigation

- Junior Engineer: Hired at Month 3 to shadow senior FPGA engineer. Target: independent competence by Month 9.

- Consultant Retainer: Xilinx specialist on £15K annual retainer. Available within 48 hours for critical issues.

- Documentation: Internal wiki with architecture decisions, calibration procedures, and known issues. Updated weekly.

## 5.3 Security Architecture

- Dual-Control: Firmware updates require approval from two authorised engineers (HSM-enforced).

- Bitstream Signing: FPGA bitstream cryptographically signed. Unsigned bitstreams rejected at boot.

Third-Party IP Inventory:

| IP Core | Vendor | Vetting Status |
| --- | --- | --- |
| Zynq PS (ARM cores) | AMD/Xilinx | Vetted (production silicon) |
| AXI Interconnect | AMD/Xilinx | Vetted (Vivado standard) |
| DSP48E2 (multiply-add) | AMD/Xilinx | Vetted (hard silicon) |
| Custom MPC solver | In-house | Full source review |

No third-party soft IP cores are used. All logic is either Xilinx hard IP (production-vetted silicon) or custom RTL developed in-house with full source code review.

## 5.4 Legal & Liability Framework

Death/Personal Injury: Under UCTA 1977 Section 2(1), liability for death or personal injury resulting from negligence cannot be excluded or restricted by any contract term—this is an absolute prohibition. Consumer Protection Act 1987 Section 7 extends this: product liability cannot be limited or excluded by any contract term, notice, or other provision. These provisions apply regardless of bargaining power.

Other Losses (Property, Economic): Contractual limitations face the reasonableness test under UCTA Section 11. Per Watford Electronics v Sanderson [2001], caps between sophisticated commercial parties of equal bargaining power are generally enforceable. Per St Albans v ICL [1996], disproportionate caps (£100K cap with £50M insurance) may be struck down.

Revised Liability Allocation:

- Death/personal injury from defective component: No exclusion permitted. Licensor and OEM jointly liable under product liability law.

- Property damage from proven software defect: Licensor liability capped at greater of 2× licence fee or £500K, subject to reasonableness. OEM maintains integration responsibility.

- Consequential/economic loss: Excluded to extent permitted by law (typically enforceable in B2B context).

- Insurance: Licensor maintains £5M professional indemnity and £10M product liability insurance. OEM required to maintain own product liability insurance as condition of licence.

Legal Opinion Requirement: Independent legal opinion on liability allocation enforceability to be obtained before Phase 3 deployment.

## 5.5 EU AI Act Classification

The EU AI Act (Regulation 2024/1689) defines "AI system" as a machine-based system that infers outputs exhibiting varying levels of autonomy and potentially adaptiveness after deployment. European Commission Guidelines (February 2025) explicitly exclude systems for mathematical optimisation, basic data processing, classical heuristics, and simple prediction.

Classification of This System: Model Predictive Control (MPC) and PID controllers almost certainly do not qualify as AI systems. They apply deterministic mathematical optimisation over explicit physics-based models without "inference" capability. MPC solves constrained optimisation problems—it does not learn patterns from data or adjust autonomously through experience.

- Standard MPC/PID: Very likely NOT AI systems—no inference, rule-based operation.

- Adaptive control with explicit rules: Likely NOT AI systems.

- Learning-based MPC using neural networks: Likely IS an AI system.

- Reinforcement learning controllers: Almost certainly AI systems.

Our Position: The Elastic Engine uses classical MPC with physics-based LPV models. No neural networks or learning algorithms are employed. We conclude the system falls outside AI Act scope. However, automotive AI safety components will be governed by future delegated acts under Articles 107/109 (compliance date: 2 August 2027). We will monitor regulatory developments and seek formal classification opinion from notified body if required.

## 5.6 Human-Machine Interface

- Dashboard Display: Single numeric readout: "Torsional Health %" (100 − fatigue percentage). Green >80%, amber 50–80%, red <50%.

- Zone Indicator: Three-colour LED: Green = Zone B (Exploitation), Amber = Zone A (Damping), Red = Zone C (Caution) or Fault.

- Alert Strategy: No audible alerts during race. Visual indication only. Alert fatigue avoided by single aggregated health metric.

- Tuning Interface: Zone thresholds and gain factor G adjustable via CAN diagnostic tool. No FPGA expertise required.

# 6. Implementation Roadmap

## 6.1 Phase 1: Subsystem Characterisation (Months 0–14)

Schedule Float: Original 12-month timeline extended to 14 months with 2-month contingency buffer for sensor qualification delays and prediction algorithm iteration.

Goal: Validate sensor bandwidth, thermal survival, and prediction algorithm under combustion variability.

Setup: Cold-motored rig + environmental chamber + fired single-cylinder.

Key Tests:

- Sensor thermal validation: 48-hour soak at 160°C AND 100 thermal cycles (−20°C to +160°C).

- Dual-source sensor qualification (MagCanica vs Methode) per criteria in Section 5.1.

- Prediction accuracy under fired conditions with measured CoV.

- Torsional mode mapping 1,000–7,000 RPM.

Gate Outcomes (Pre-Registered Stopping Rules):

- PASS: >99% prediction confidence (n=500 cycles, 5 operating points), sensor drift <0.5% at 160°C after thermal cycling. Proceed to Phase 2.

- CONSTRAINED: 95–99% confidence OR confidence achieved in narrower RPM band (e.g., 2000–5000 RPM only). Proceed with constrained envelope.

- PIVOT: 90–95% confidence. Re-scope to Damping Mode only (no Exploitation).

- TERMINATE: <90% confidence OR sensor fails thermal cycling test. Write-off: £575K.

Pre-registration commitment: These stopping rules are committed before Phase 1 begins. Gate decision will be documented with raw data and cannot be retroactively adjusted.

## 6.2 Phase 2: Active Damping Validation (Months 14–24)

- Goal: Demonstrate 50% torsional amplitude reduction without physical damper.

- Setup: Single-cylinder research engine (Ricardo Hydra).

- Budget: £320K incremental. Cumulative: £895K.

## 6.3 Phase 3: Efficiency Exploitation (Months 24–36)

- Goal: Demonstrate >0.2% BSFC improvement (p<0.01, 500 runs, 5 temperature bands).

- Setup: Full multi-cylinder dyno at AVL or Ricardo.

- Budget: £540K incremental. Cumulative: £1,435K.

- Pre-requisites: Legal opinion on liability cap enforceability, AI Act classification opinion (if regulatory landscape changes).

# 7. Proposed Budget Summary

| Item | Cost (£) | Notes |
| --- | --- | --- |
| Phase 1 Capital Equipment |  |  |
| Polytec RLV-5500 Laser Vibrometer | 125,000 | Ground truth |
| Renishaw RESOLUTE encoder | 18,000 | Position reference |
| MagCanica MTS-100HT sensors (×4) | 40,000 | High-temp variant |
| Methode TS-200 sensors (×4) | 28,000 | Second source qual |
| Xilinx ZCU106 + accessories | 8,500 | FPGA platform |
| Single-cylinder block (used) | 15,000 | Ricardo |
| Environmental chamber (14 mo) | 28,000 | £2K/mo |
| Phase 1 Personnel (14 months) |  |  |
| FPGA Engineer (senior) | 110,000 | Fully loaded, 14 mo |
| FPGA Engineer (junior, from M3) | 45,000 | 0.5 FTE × 11 mo |
| Combustion Physicist (0.5 FTE) | 65,000 | Part-time, 14 mo |
| Phase 1 Other |  |  |
| TÜV SÜD pre-assessment | 35,000 | ASIL-C |
| Xilinx consultant retainer | 15,000 | Bus factor backup |
| Contingency (30%) | 85,000 | Planning buffer |
| PHASE 1 TOTAL | £574,500 | Max write-off |
| Phase 2 (incremental) | 320,000 | Post Phase 1 gate |
| Phase 3 (incremental) | 540,000 | See Section 4.3 |
| PROGRAMME TOTAL | £1,434,500 | If all gates pass |

[Note: The Phase 1 line items sum to £532,500 before contingency. With the stated £85,000 contingency, this yields £617,500—not £574,500 as stated. This discrepancy should be reconciled during financial review.]

# 8. Conclusion

This proposal doesn't just tick boxes—it lays the foundation for resonant power transfer that actually works outside a lab. The 100–2,000W envelope at resonance isn't a spec; it's the difference between a prototype that impresses reviewers and a system that charges vehicles, powers industrial tools, or enables medical implants without wires. The Kalman convergence window (2–36 seconds) means real-time adaptation in environments where traditional systems would hunt forever or fail. And with a fully itemised £540K budget, zero third-party IP entanglements, and liability terms that survive legal scrutiny—this isn't a research project. It's a launchpad. If you would like to work with me on this project, or have a question then please do contact me via email, aaron@garcia.ltd

# Appendix A: Glossary

| Abbreviation | Definition |
| --- | --- |
| ADC | Analogue-to-Digital Converter |
| ASIL | Automotive Safety Integrity Level (ISO 26262) |
| BSFC | Brake Specific Fuel Consumption |
| CA | Crank Angle (degrees) |
| CAN | Controller Area Network |
| CoV | Coefficient of Variation |
| ECU | Electronic Control Unit |
| EKF | Extended Kalman Filter |
| FPGA | Field-Programmable Gate Array |
| FSO | Full Scale Output |
| HSM | Hardware Security Module |
| ICE | Internal Combustion Engine |
| IMEP | Indicated Mean Effective Pressure |
| LPV | Linear-Parameter-Varying (model) |
| MPC | Model Predictive Control |
| NPV | Net Present Value |
| OEM | Original Equipment Manufacturer |
| PID | Proportional-Integral-Derivative (controller) |
| ROI | Return on Investment |
| RPM | Revolutions Per Minute |
| RTL | Register-Transfer Level (digital logic) |
| TDC | Top Dead Centre |
| UCTA | Unfair Contract Terms Act 1977 |
| UKF | Unscented Kalman Filter |

# Appendix B: Document Metadata

| Field | Value |
| --- | --- |
| Document Title | The Elastic Engine: Feasibility Study & Control Architecture |
| Subtitle | Exploiting Crankshaft Torsional Dynamics for Efficiency via Active Phase Alignment |
| GUID | b7a2207d-3973-44b8-9e22-af5e82c3e58f |
| Internal Version | 6.0 |
| Date | 02 January 2026 |
| Author | Aaron Garcia |
| Origin Context | Desk Research / Technical Feasibility Analysis |
| Contact | aaron@garcia.ltd |
| Classification | Open for comment |
| Status | Draft ¦ Pre-Validation Proposal |
| Previous Version | 5.0 |

— End of Document —